# Number Bases Exercise Sheet - SHOW ALL WORKING

Give all Binary answers in 8 bits and all Hexadecimal answers in 4 bit.

- Write down and label the positive integers from 0 – 15 in 4 bit binary

- Convert the following positive integers.

  - 111110002  to base 10

  - FC0516 to base 10

  - 009E16 to base 2

  - 12710 to base 2

  - 409510 to base 16

  - 000F16 to base 2

- Write down and label the signed integers (i.e. +/-) from –8 to +7 in 2’s Compliment.

- How many bits make a byte?

- How many bits make a Hexadecimal digit?

- How many Hexadecimal digits is 1 byte?

- Add the following unsigned (positive) integers in 8 bit binary and show all working.

  - 001011112 + 100100102

  - 5010 + 6010

  - 111000112 + 001001002

- Perform the following subtraction using 2’Comlpiments where required. Work in 4 or 8 bit as required.

  - 410 – 210

  - 8 – 5

  - 010010112 + 100111112

- What is the 2’s compliment of  01110000

Base 2 (Binary) Conversion Table - 8bit or 1 byte – Range 0-255

| 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |

Base 16 (Hexadecimal) Conversion Table - 32bit or 4 bytes – 
Range 0 – 4 294 967 296 (4Gig)

| 167 | 166 | 165 | 164 | 163 | 162 | 161 | 160 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 268 435 456 | 16 777 216 | 1 048 576 | 65 536 | 4 096 | 256 | 16 | 1 |