Find the broadcast address for the following IP addresses?

- 192.168.5.42/28

- 10.0.255.164/26

- 7.30.255.56/8

Which of the following are on the same network?

192.168.255.18/28

192.168.255.71/28

192.168.255.250/28

192.168.255.30/28

192.168.255.66/28

Find the Network Block for

165.255.255.120/29

Subnet Mask:

Network Address:

First IP Address:

Last IP Address:

Broadcast Address:

What is wrong with the following IP Address – Why?

172.16.254.160/27

Hub PC Name: Room1_PC3 Workgroup: BLD4_LVL1 IP address: 212.10.2.60 Subnet Mask: 255.255.255.240 PC Name: Room1_PC2 Workgroup: BLD4_LVL1 IP address: 212.10.2.47 Subnet Mask: 255.255.255.240 PC Name: Room1_PC1 Workgroup: BLD4_LVL1 IP address: 212.10.2.50 Subnet Mask: 255.255.255.240 Router

You have been allocated a Class A IP address from your Network Administrator and have been requested to configure a 3 LANs for an organization.

The network address is – 67. 9. 9. 0 / 24

How many valid sub-networks will you get? ___________

What will be the new Subnet Mask?      _______ . _______ . ________ . ________

List the total available Subnet addresses.  	(*Note the table size is not a clue!)

| Binary Subnet | Complete the sub network, network address in dotted decimal notation for each sub network. | Complete the sub network broadcast address in dotted decimal notation for each sub network. | Configurable host IP ranges for each sub network. | Valid? Yes or No |
| --- | --- | --- | --- | --- |

How many hosts will you be able to configure to each Sub-network?

You have been allocated a Class B IP address from your Network Administrator and have been requested to configure a 17 LANs for an organization.

The network address is – 167. 255. 254. 0 / 24

How many valid sub-networks will you get? ___________

What will be the new Subnet Mask?      _______ . _______ . ________ . ________

List the total available Subnet addresses.  	(*Note the table size is not a clue!)

| Binary Subnet | Complete the sub network, network address in dotted decimal notation for each sub network. | Complete the sub network broadcast address in dotted decimal notation for each sub network. | Configurable host IP ranges for each sub network. | Valid? Yes or No |
| --- | --- | --- | --- | --- |

How many hosts will you be able to configure to each Sub-network?