Find the broadcast address for the following IP addresses?

- 192.168.5.42/28      Borrowed = 4, Remaining  = 4

Block = 2^4 = 16

Therefore networks on 		.0	.16	.32	.48	.64 	…

192.168.5.42 is on the .32 network so the broadcast address is one less than the next network = 192.168.5.47

- 10.0.255.164/26	Borrowed =2, Remaining = 6

Block = 2^6 = 64

Therefore networks on 		.0	.64	.128 	.192

10.0.255.164 is on the .128 network so the broadcast address is one less than the next network = 10.0.255.191

- 7.30.255.56/8     8 bit mask  (Remember 8, 16, 24)  1st Octet is network.

7.30.255.56  is on the network 7.0.0.0  so the broadcast address is one less than the next network = 7.255.255.255

Which of the following are on the same network?

Block = 2^4 = 16 so networks are	.0  .16  .32  .48  .64  .80  .96  .112  .128  .144  .160  .176  .192  .208  .224  .240

192.168.255.18/28		.16 network

192.168.255.71/28		.64 network

192.168.255.250/28		.240 network but invalid!

192.168.255.30/28		.16 network

192.168.255.66/28		.64 network

Find the Network Block for

165.255.255.120/29 Borrowed = 5 Remaining = 3

Block = 2^3 = 8 so networks are	.0  .8  .16  .24  .32  .40  .48  .56  .64  .72  .80  .88  .96  .104  .112  .120  .128

Subnet Mask: 255.255.255.248

Network Address: 165.255.255.120

First IP Address: 165.255.255.121

Last IP Address: 165.255.255.126

Broadcast Address: 165.255.255.127

What is wrong with the following IP Address – Why?

172.16.254.160/27 Borrowed = 3 Remaining = 5

Block = 2^5 = 32 so networks are	.0  .32  .64  .128  .160

It is not an IP Address it is a Network Address

.240 mask is 1111000       Borrowed = 4 Remaining = 4

Block = 2^4 = 16 so networks are	.0  .16  .32  .48  .64  .80  .96  .112  .128  .144  .160  .176  .192  .208  .224  .240

Hub PC Name: Room1_PC3 Workgroup: BLD4_LVL1 IP address: 212.10.2.60 Subnet Mask: 255.255.255.240 PC Name: Room1_PC2 Workgroup: BLD4_LVL1 IP address: 212.10.2.47 Subnet Mask: 255.255.255.240 PC Name: Room1_PC1 Workgroup: BLD4_LVL1 IP address: 212.10.2.50 Subnet Mask: 255.255.255.240 Router This machine is on a broadcast address – Move it to a valid range with the other PCs, perhaps 212.10.2.55

You have been allocated a Class A IP address from your Network Administrator and have been requested to configure a 3 LANs for an organization.

The network address is – 67. 9. 9. 0 / 24  We need to borrow 3 bits to get at least 3 lans

How many valid sub-networks will you get?     2^3 – 2 = 6_________

What will be the new Subnet Mask?      255.255.255.224

List the total available Subnet addresses.  	(*Note the table size is not a clue!)

| Binary Subnet | Complete the sub network, network address in dotted decimal notation for each sub network. | Complete the sub network broadcast address in dotted decimal notation for each sub network. | Configurable host IP ranges for each sub network. | Valid? Yes or No |
| --- | --- | --- | --- | --- |
| 000 | 67.9.9.0 | 67.9.9.31 | .1 - .30 | N |
| 001 | 67.9.9.32 | 67.9.9.63 | .33 - .62 | Y |
| 010 | 67.9.9.64 | 67.9.9.95 | .65 - .94 | Y |
| 011 | 67.9.9.96 | 67.9.9.127 | .97 - .126 | Y |
| 100 | 67.9.9.128 | 67.9.9.159 | .129 - .158 | Y |
| 101 | 67.9.9.160 | 67.9.9.191 | .161 - .190 | Y |
| 110 | 67.9.9.192 | 67.9.9.223 | .193 - .222 | Y |
| 111 | 67.9.9.224 | 67.9.9.255 | .225 - .254 | N |

How many hosts will you be able to configure to each Sub-network?

There are 5 bits in the node portion so you would get 2^5-2 hosts on each network = 30